================ Xem số Ip đang kết nối ================================================

/ip firewall address-list remove [find list=active-443-now]

:foreach c in=[/ip firewall connection find where protocol=tcp and dst-address~"222.254.35.122:443"] do={

:local src [/ip firewall connection get $c src-address]

:local p [:find $src ":"]

:local ip $src

:if ($p != nil) do={

:set ip [:pick $src 0 $p]

}

:if ([:len [/ip firewall address-list find list=active-443-now address=$ip]] = 0) do={

/ip firewall address-list add list=active-443-now address=$ip timeout=5m comment="auto count active 443"

}

}

:put ("Tong so IP khac nhau dang truy cap 443: " . [/ip firewall address-list print count-only where list=active-443-now])

/ip firewall address-list print where list=active-443-now

=====================================================================================